- Feature Name: Kurtosis Burst Index
What it shows: Detects early-stage faults via transient spikes. High values indicate intermittent impacts from incipient defects.
How to calculate: % of segments with kurtosis > 5 (healthy bearings typically show 3-4).

- Feature Name: RMS Trend Slope
What it shows: Progressive degradation monitoring. Sustained positive slopes indicate worsening defects.
How to calculate: Linear regression slope of segment RMS values over time.

- Feature Name: Peak-to-RMS Ratio SD
What it shows: Signal instability. High variability suggests inconsistent loading or noise interference.
How to calculate: Standard deviation of (Peak/RMS) across all segments.

- Feature Name: Degradation Acceleration
What it shows: Critical fault progression. Identifies when degradation rate increases exponentially.
How to calculate: Quadratic coefficient from polynomial fit to RMS values over segment index.

- Feature Name: Transient Event Density
What it shows: Sudden damage (cracks/spalls). Flags clustered high-energy events.
How to calculate: % of segments where (Peak > 5 × median segment Peak).

- Feature Name: Crest Factor Stability
What it shows: Bearing health consistency. Low values suggest stable operation; drops may indicate developing faults.
How to calculate: 1 / (σ of segment Crest Factors + ε) where ε prevents division by zero.

- Feature Name: Kurtosis-RMS Covariance
What it shows: Early fault confirmation. Negative correlation (high kurtosis + low RMS) indicates initial defects.
How to calculate: Cov(Kurtosis, RMS) across segments.

- Feature Name: Fault Progression Score
What it shows: Composite health metric. Combines multiple indicators for holistic assessment.
How to calculate: (Max Peak × Kurtosis Burst Index) / (RMS Trend Slope + ε).

- Feature Name: Skewness Polarity Shift
What it shows: Asymmetric damage development. Positive shifts indicate impact-dominated faults.
How to calculate: % of segments with skewness > 0.5 minus % with skewness < -0.5.

- Feature Name: Top 3 Segment Severity
What it shows: Worst-case damage assessment. Highlights critical transient events.
How to calculate: Average of (Peak × Kurtosis) for the 3 segments with highest Peak values.

- Feature Name: Noise Contamination Index
What it shows: Signal quality issues. High values indicate electromagnetic interference or sensor problems.
How to calculate: % of segments with (RMS < 0.1 × overall median RMS) AND (Peak > 3 × segment RMS).

- Feature Name: Impulse Persistence
What it shows: Bearing fault consistency. Distinguishes random noise from repeating defects.
How to calculate: Max consecutive segments with kurtosis > 4.5, normalized by total segments.

- Feature Name: Dynamic Range Collapse
What it shows: Advanced degradation. Indicates loss of signal variability during failure modes.
How to calculate: (Max segment Peak - Min segment Peak) / overall RMS.

- Feature Name: Trend Inconsistency Flag
What it shows: Unstable operating conditions. Detects erratic behavior from misalignment or lubrication issues.
How to calculate: 1 if RMS trend R² < 0.7, else 0.

- Feature Name: Harmonic Distortion Indicator
What it shows: Nonlinear system changes. Suggests developing cracks or brinelling.
How to calculate: Skewness of segment RMS values multiplied by kurtosis of segment Peaks.

Key Engineering Insights:

- Early Fault Detection: Kurtosis Burst Index + Kurtosis-RMS Covariance target initial defects before energy increases.

- Degradation Tracking: RMS Trend Slope + Degradation Acceleration provide time-to-failure estimates.

- Noise Immunity: Noise Contamination Index helps distinguish true faults from measurement artifacts.

- Critical Event Capture: Top 3 Segment Severity prevents transient events from being averaged out.

- Operational Context: Trend Inconsistency Flag accounts for non-bearing related variations.

These features enable compressed yet comprehensive signal assessment, balancing statistical robustness